DEVELOPMENT NAME:

DEVELOPMENT LOCATION:

| ITEM | VERIFICATION (Yes/No/NA) | COMMENT |
| --- | --- | --- |
| STORMWATER DRAINAGE SYSTEM |  |  |
| - Pipework has been visually inspected and is satisfactory, ie:, - free of debris and siltation - pipe joints satisfactory with no deflection or movement - no visible sign of trench subsidence - no exposed reinforcing steel to cut pipe ends |  |  |
| - Gully pits and manholes have been visually inspected and are satisfactory, ie., - no ponding - no excessive cracking or distress of reinforced concrete works - clear of silt and debris - all mortar is in place, no excessive spalling, flaking or cracking - no visible sign of subsidence |  |  |
| - Gross Pollutant traps have been visually inspected and are satisfactory, ie., - free of debris and siltation - no cracking or distress of concrete at fixing points - fasteners are secure - structures have not misaligned due to excessive loads - no corrosion at any location evident |  |  |
| - overland flow paths clear. |  |  |
| ALLOTMENT DRAINAGE |  |  |
| - Concrete catch drains have been visually inspected and are satisfactory, ie., - clear of silt and debris - no damage or cracking - overland flow path profile maintained |  |  |
| EARTHWORKS/SITE WORKS |  |  |
| - Sediment / erosion control measure maintained. |  |  |
| - All batter slopes stable and no distress exhibited. |  |  |
| - 80% grass cover achieved. |  |  |
| - Street tree planting is satisfactory. |  |  |

| ITEM | VERIFICATION (Yes/No/NA) | COMMENT |
| --- | --- | --- |
| WATER RETICULATION |  |  |
| - No visible signs of trench subsidence. |  |  |
| - Valves and hydrants have been inspected and are satisfactory, ie., - no leaks - valve and hydrant markings - no damage |  |  |
| SEWERAGE RETICULATION |  |  |
| - No visible signs of trench subsidence. |  |  |
| - Pipework has been visibly inspected and is satisfactory, ie., - alignment satisfactory - clear of silt and debris (flushed) - no ponding - pipework not oval or compressed. |  |  |
| - Manholes/structures have been visually inspected and are satisfactory, ie., - benching no signs of cracking, spalling ok - no weeping or infiltration - no ponding or disposition of solids. |  |  |
| ROADWORKS |  |  |
| - Road pavement has been visually inspected and satisfactory, ie., - no damage to Wearing Course - no ponding - clear of siltation and debris |  |  |
| - Kerb and channel has been visually inspected and is satisfactory, ie., - no excessive cracking or distress to concrete works - no ponding - service conduit markers ok - no differential settlement or dislocation of pavement surface and concrete kerb and channel. |  |  |
| - Linemarking and road signage satisfactory. |  |  |
| MISCELLANEOUS |  |  |
| - Footpaths and concrete works satisfactory. |  |  |
| - Bikeways and associated works satisfactory. |  |  |
| - Street name signage satisfactory. |  |  |
| - Alternative pavement surfacing (eg pavers, stamped concrete) is satisfactory. |  |  |
| OTHER MATTERS |  |  |
| - Specific matters in relation to the site. |  |  |
| INSPECTING ENGINEER’S SIGNATURE: NAME: CONSULTING ENGINEER: RPEQ No: DATE: / / |  |  |